Created:

15.07.2019

Contact:

Erlend Kvinnesland
erlend.kvinnesland@geocap.no
+47 415 60 664
Geocap AS

Maritime Limits and 
  Boundaries for ArcGIS Tutorial

Tutorial

Contents

Introduction	2

About the extension	2

Projections and geodetic calculations	2

Normal and Straight baselines	2

Training data	3

Getting started	4

Setting up the extension	4

Demo data	4

Geoprocessing options	6

Geoprocessing Results	6

Limit lines	7

Input	7

Output	8

Basics	10

Limit calculation area	11

Multiple distance calculations	12

Normal and straight baselines in one dataset	13

Limit input to the calculation	14

Multiple input calculation	15

Equidistance lines	17

Input	17

Output	18

Straight baselines	19

We are now ready to start calculating equidistance lines	20

Basic calculation	20

Multiple states calculations	21

Weighted calculations	22

Area calculation	23

# Introduction

## About the extension

Maritime Limits and Boundaries (MLB) for ArcGIS is an extension which provides a set of geodetic tools for calculating maritime limit lines and maritime boundary lines in ArcMap. The extension has been developed by Geocap AS in collaboration with Geoscience Australia and Canadian Hydrographic Service.

Maritime boundaries form the outermost limits for most States and their accurate computation is a matter of national priority. When a State realizes and releases its maritime boundary information it increases the potential for international harmony, economic activity, and environmental protection. Precisely calculated digital maritime boundary information reduces ambiguity in the spatial definition of a State’s maritime zones.

The Maritime Limits and Boundaries for ArcGIS extension effectively minimizes the regulatory overhead of maintaining a digital dataset of maritime limits and boundaries. The solution can calculate large datasets without the necessity of datum or data format transformations.

The extension consists of geoprocessing tools for:

- Limit line calculations – which will precisely calculate the Territorial Sea, Contiguous Zone and Exclusive Economic Zone of a State. In fact, it will calculate any user specified geodesic distance from the baseline.

- Midline/Equidistant line calculation – which makes it easy to accurately calculate different maritime boundary scenarios between two or more States.

When referring to the Maritime Limits and Boundaries extension and tools we often use the abbreviation MLB.

## Projections and geodetic calculations

The MLB tools use geodesic operations and the resulting lines will have a high accuracy. If the source/input data is projected the tools will automatically convert the data to geodetic coordinates, perform the computation and convert the result to the projection of the source/input data. This means your source data can have any coordinate system.

## Normal and Straight baselines

A baseline, as defined by the United Nations Convention on the Law of the Sea, is the line along the coast from which the seaward limits of a maritime zone of jurisdiction is measured. Normally, a baseline follows the low-water line of a coastal state. When the coast is deeply indented, has fringing islands or is highly unstable, straight baselines may be used.

Different algorithms must be used when calculating maritime limits from normal and straight baselines. The benefit of using the Maritime Limits and Boundaries extension from Geocap is the you can do both computations in one operation. The tools will automatically distinguish between normal and straight segments and use the appropriate algorithm.

There are different ways of doing this depending on how your baseline data is structured. This tutorial will go through these different options.

## Training data

The training data used in this tutorial is a made-up dataset called Atlantis. It consist of one coastline dataset for the whole of Atlantis, and then there are three different baseline datsets. One for each State. East Atlantis consist of only straight baselines, South Atlantis consist of a normal baseline, while North Atlantis consist of both normal and straight baseline segments. In addition to this there is an Area of Interest dataset which is approximately 200NM from the coastline. This polygon dataset has been split into three different regions.

All input feature classes are stored in a file geodatabase. During the training you will also write resulting feature classes to the same database.

# Getting started

The Maritime Limits and Boundaries extension is provided as an install shield (.exe). If the extension has not already been installed on your computer and you need to get hold of the .exe-file, please contact support@geocap.no

## Setting up the extension

Once the extension has been installed you need to activate it and add the tools.

#### Exercise 1 - Activate the Maritime Delimitation extension

- In the top menu in ArcMap, click Customize, then Extensions.

- Tick the box Geocap Maritime Delimitation

- Click Close

#### Exercise 2 - Add the Maritime Delimitation tools

- In the Catalog window expand Toolboxes

- Right click My Toolboxes, go to New and click Toolbox

- Right click the new Toolbox and click Rename

- Type in the name Maritime Limits and Boundaries

- Right-click the new toolbox, select Add, and click Tool....

- Expand the toolbox Geocap Tools.

- Check the box next to Maritime Delimitation and click OK.

The Geocap Maritime Delimitation tools are now added to your toolbox.

## Demo data

The demo data is available in a file geodatabase. To make it easy to work with this data we can set up a folder connection where the file geodatabase is stored.

#### Exercise 3 – Connect to the demo database

- In the Catalog pane, click the Connect To Folder button

- Browse and select the folder with the Atlantis_MLB database and click OK

- Observe that the folder has been added under Folder Connections

#### Exercise 4 – Display data

- Under Folder Connections in the Catalog pane, expand the Atlantis_MLB database

- Click and drag the following feature classes into the map

  - The_Kingdom_of_Atlantis_Coastline

  - North_Atlantis_Baseline

  - South_Atlantis_Baseline

  - East_Atlantis_Baseline

  - AOI

- Try turning layers on and off in the Table of Content

To find datasets more easily in the table of content (TOC) it is useful to group datasets together, i.e. the baseline datasets

#### Exercise 5 – Group baseline datasets

- Hold down Ctrl on your keyboard and click on the following datasets:

  - North Atlantis Baseline

  - South Atlantis Baseline

  - East Atlantis Baseline

- Right click and select Group

- Rename New Group Layer to Baselines

To identify different datasets in the map more easily it is useful to set up proper symbology for the different datsets.

#### Exercise 6 – Change symbology

- In the TOC click on the line symbol under North_Atlantis_Baseline

- Change Color to red and the Width to 2 in the Symbol Selector menu

- Click OK

- Do the same with South_Atlantis_Baseline and East_Atlantis_Baseline, but use different colors

- Click on the rectangle symbol under Area of Interest

- Set Fill Color to No Color in the Symbol Selector menu

#### Exercise 7 – Save project

- Click File and then Save

- Navigate to the same folder as the Atlantis_MLB database is stored

- Type in the filename Atlantis.mxd

- Click Save

We have now set up the demo dataset and we are ready to calculate limits and boundaries, but before we start working with the geoprocessing tools it is useful to know a bit more about how to work with the geoprocessing tools.

## Geoprocessing options

The Geoprocessing Options menu provides options related to the geoprocessing tools you run in ArcMap. Some important options when using the Maritime Limits and Boundaries tools are

Overwrite the outputs of geoprocessing operations
The check box controls whether tools automatically overwrite any existing output, if the output name is the same. When it is checked, you receive a warning before tool execution that the output exists, but the tool executes and overwrites the output dataset. With this option off, existing outputs are not overwritten, and the tool displays an error, preventing you from executing the tool.

We recommend to have this option turned on. Especially when trying out different parameters and you don`t want to fill up your database with too many datasets.

Background Processing

The Background processing panel is where you control whether a tool executes in foreground or background mode.

- If Enable is checked, tools execute in the background, and you can continue working with ArcMap while the tool executes. You'll see a progress bar at the bottom of your document displaying the name of the currently executing tool. When the tool finishes executing, a pop-up notification appears on the system tray. You can control how long the notification message appears by moving the Notification slider or disable the message by moving to the far left.

- If Enable is not checked, tools execute in the foreground. When a tool is executing in foreground mode, you must wait until the tools stop executing before you can continue with other work.

Since some of the geoprocessing tools can take a while to run we recommend turning background processing on. However, if you have background processing turns on and have installed 64-bit background processing, the tools will not work.

## Geoprocessing Results

Whenever you execute a tool using its dialog box, information about the execution is written as a result in the Results window. There are several useful things you can do with results in the Results window, such as:

- Open a previous execution of a tool to change a few parameters without having to reenter all parameters, then rerun.

- View the status and messages of a tool executing in the background

The Results window maintains information between application sessions. That is, you can retrieve information about a tool you executed weeks ago. Results are kept for a period as defined in the geoprocessing options. The default is to remove results older than two weeks.

# Limit lines

The Calculate Limit Line tools calculate maritime limits based on a given baseline and distance. The two tools "Calculate Limit Line" and "Calculate Limit Line Multiple Input" will produce the same output. The only difference between the two tools is how the input baseline is organized. Both tools support a combination of "Straight" and "Normal" baselines. The tools use geodesic operations and the resulting limit line will have a high accuracy.

## Input

#### Exercise 1 – Investigate the input parameters of the Limit Line calculation tool

- Double click the Calculate Limit Line tool

- Click Show Help >>

- Click on each input element and read the help text

- Click Cancel

About Point spacing along Distance Line

The arc on an "envelope of arcs" in reality consist of a series of short chords. The point spacing represents the length of these chords. In the image below, you will see that a larger area will be lost (outside blue chords) if we double the point spacing on the arc.

It is recommended that the chord lengths (point spacing) are determined such that the distance between the arc and the chord never exceeds 1 meter. Below you will find a table of distance lines and the corresponding point spacing which fulfills this recommendation. The tools will automatically suggest these distances, but it can be overwritten by the user.

| Distance line (M) | Point spacing (M) |
| --- | --- |
| 3 Nautical Miles | 0.1 Nautical Miles |
| 12 Nautical Miles | 0.2 Nautical Miles |
| 60 Nautical Miles | 0.5 Nautical Miles |
| 200 Nautical Miles | 1.0 Nautical Miles |
| 350 Nautical Miles | 1.0 Nautical Miles |

## Output

The Calculate Limit Line tools offer four different output datasets. It is up to the user to decide which output dataset to generate. The name of the resulting dataset will use the user defined output name as the prefix and the i.e _Connection_Polygons.

Read more about the different output datasets below:

Output Distance Line
Consist of many features where each feature is the part of the limit line which share the same base point, or same straight baseline. If the limit line feature is built from a normal baseline, the feature geometry will be an arc. For limit line features build from a straight baseline the geometry is a made using the method of "tracés paralléles" from the straight baseline.

All features will meet the neighboring features in a common point. This common point is accurately calculated using geodesic calculations. This can be where two arcs cross if both base line points are normal baselines. If the baseline has a combination of straight and normal base lines, it can also be a crossing between an arc and a line calculated using the method of "tracés paralléles", or the crossing of two lines using the method of "tracés paralléles".

The feature table of a limit line will have columns for the original feature class, the original feature object id, the critical points object id, and more.

Output Distance Polygon
Is a combination of the "Calculation Area Polygon" and the generated Limit line. In order for this to be useful, you should spend some time to carefully set up the calculation area polygon. The "inner part" of the calculation area could be a coast line, or the base line. It may have holes if you do not want to include islands. The calculation area could also include boundaries against neighboring states. The algorithm will treat the line pieces of the calculation area polygon as geodesics. If this is not the case, you should use the "Densify" or "Geodetic Densify" tool on the border line piece before you include it in the calculation area polygon.

Output Critical Points
Consist of the baseline points that have been used to create the limit line. The limit line will have a column with an object id to these points. They will also have a database "relation". The critical point will also have the feature class name, object id, and a relation to the original baseline feature. In addition, it has columns with the latitude and longitude of the point.

Output Connection polygon
Is used to visualize the association between the limit line and the critical baseline points. In the connection polygon feature class, each feature is a combination of a limit line piece, and the associated base line point or normal base line. The geometry of a connection polygon feature from a normal baseline point will look like a circular sector. The geometry of a connection polygon from a straight baseline will look like a rectangle.

## Basics

#### Exercise 3 - Basic limit calculation

- Double click the Calculate Limit Line tool

- From the dropdown menu select North Atlantis Baseline

- Under Output name type in EEZ

- Leave all other options as default and click OK

- Once the tool finishes the results should automatically appear in the map

#### Exercise 3 – Investigate results

- Turn off EEZ_Distance_Polygon

- Take a closer look at the EEZ_Connection_Polygons and notice that each segment of the line points back to the basepoint which was used to generate it.

- Take a closer look at the EEZ_Critical_Points. This is a subset of North Atlantis Baseline which only contains points which contributes to the EEZ. Often called critical points.

Related Tables

The Limit Line tools uses Related Tables to link source data to the results. Note that the source data and result need to be in the same database for this to work.

Read more about Related Tables here:
http://desktop.arcgis.com/en/arcmap/10.6/manage-data/tables/essentials-of-relating-tables.htm

#### Exercise 4 – Investigate metadata

- Turn off EEZ_Connection_Polygons

- Right click EEZ_Distance_Lines and select Open Attribute Table

- Notice the Baseline_type column which only contains normal at this point. Once we introduce straight baselines this will change.

- Notice the CriticalPoint column which refers to the objectID in the EEZ_Critical_Points dataset

- Right click EEZ_Distance_Lines,  go to Selection and click Make this the only selectable layer

- On the main Toolbar click the Selection button  and select a portion of the EEZ_Distance_Lines dataset in the map.

- In the Attribute Table, click the Related Tables buttons   and select EEZ_Critical_Points_to_EEZ_Distance_Lines_relationship

- Notice that the Attribute Table for EEZ_Critical_Points is opened automatically and shows the basepoint(s) which were used to generate the portion of the distance line you selected.

- Now try selecting a critical basepoint and show the associated line segment it created.

Tip: You might have to zoom out in the map to see both the critical point and line segment at the same time.

## Limit calculation area

The Calculate Limit Lines tools have no notion about sea and land, or any other States within the calculation area. By looking at the result from your previous calculation you see that both South Atlantis and East Atlantis is ignored. To ensure we only get maritime limits at sea and take other States into account you should use an area of interest polygon as input to the limit line calculation. Note that If your area of interest consists of multiple polygons the calculation will honor any selection you make.   Another benefit of using an area of interest is that you do not have to define a coastline/baseline direction.

#### Exercise 5 - Selected area of interest calculation

- On the main Toolbar click the Selection button  and select the western part of the Area of Interest. (You might have to make Area of Interest the only selectable layer)

- Double click the Calculate Limit Line tool

- From the dropdown menu select North Atlantis Baseline

- Under Calculation Area polygon select Area of Interest

- Under Output name type in EEZ (Notice the warning about dataset being overwritten)

- Leave all other options as default and click OK

- Once the tool finishes, notice that we only get results inside the selected Area of Interest

## Multiple distance calculations

Sometimes it is useful to calculate multiple distances in one operation. This can easily be done by simply adding more distances in the Distance field, separated by whitespace. The different resulting lines will end up in the same dataset, with an attribute specifying the distance.

The point spacing on the resulting distance line will automatically be updated based on the distance.

#### Exercise 6 - Multiple zones calculation

- Double click the Calculate Limit Line tool

- From the dropdown menu select North Atlantis Baseline

- Under Distance type in 200 24 12 and click Tab on your keyboard

- Notice that Point Spacing Along Distance Line is automatically updated

- Under Calculation Area polygon select Area of Interest (Make sure the western part of the Area of Interest has been selected)

- Under Output name type in Multiple

- Leave all other options as default and click OK

- Once the tool finishes, notice that the resulting datasets contains multiple distances.

Since all distance lines are in the same dataset it might be useful to color them differently to distinguish them better.

#### Exercise 7 – Color distances differently

- Right click Multiple_Distance_Lines and click Properties

- Click the Symbology tab

- On the left side of the menu click Categories and select Unique values

- In the Value field select Distance

- Click Add All Values

- Click OK

- Notice that the distance lines are now colored differently

Should you want to split up the dataset so you get one dataset for each distance line you can use the ArcGIS tool Split By Attributes.

#### Exercise 8 - Splitting zones

- Open the tool Split By Attributes which you will find in the System Toolboxes under Analysis Tools and Extract

- Under Input Table, select Multiple_Distance_Lines

- Under Target Workspace browse to and select the Atlantis_MLB.gdb database

- Under Split Fields select Distance and click OK

- Once the tool finishes confirm that the tool has created three new datasets in the database.

## Normal and straight baselines in one dataset

When investigating the North Atlantis Baseline you will see that it consist of both normal and straight segments. For this particular dataset the straight segments are included in the baseline dataset together with the normal segments. In the attribute table there is an attribute specifying if the segment is straight or normal.

If you look closely at the 12NM distance line in the picture below you will see that the straight segments which the North Atlantis Baseline consist of, has not been taken into account in our calculations so far. We can take these into account by using the straight baseline attribute to ensure the proper algorithm is used.

#### Exercise 9 – Investigate datasets with both normal and straight segments

- Right click North Atlantis Baseline and click Open Attribute Table

- Find the Type column, scroll through the table and note that there are both normal and straight attributes.

- Note that for straight segments the VxCount only lists 2 vertices, a start point and an endpoint. Normal segments can contain a lot more points.

- Select a straight line segment by clicking in the left column of the attribute table

- Right click the same column and row again and select zoom to selected

- Observe the difference between the straight and normal segments in the map

#### Exercise 10 – Calculate from baseline with both normal and straight segments

- Make sure only the western part of the Area of Interest has been selected

- Double click the Calculate Limit Line tool

- From the Baseline Feature Class dropdown menu select North Atlantis Baseline

- From the Baseline Type Column dropdown menu select Type

- From the Straight Baseline Attribute dropdown menu select Straight

- Under Distance type in 12

- Under Calculation Area polygon select Area of Interest

- Under Output name type in TS

- Leave all other options as default and click OK

- Once the tool finishes observe that both normal and straight segments have been used to compute the limit line.

## Limit input to the calculation

In many cases it can be useful not to include all points or segments in a calculation. Instead of creating a new dataset you can simply select which points you want to calculate from, either by manual selection in the map or by attribute filtering.

#### Exercise 11 - Selected points calculation

- Make sure the western part of the Area of Interest has been selected

- In addition, select one of the line segments from the Attribute Table on the North Atlantis Baseline dataset

- Double click the Calculate Limit Line tool

- From the dropdown menu select North Atlantis Baseline (If you select a straight segment make sure you use the baseline type option)

- Under Distance type in 12

- Under Calculation Area polygon select Area of Interest

- Under Output name type in TS

- Leave all other options as default and click OK

- Once the tool finishes, notice that the resulting line is only calculated from the selected points

Definition queries

You can use definition queries to only show a limited part of a baseline, i.e containing a specific attribute. The geoprocessing tools will honor these definition queries and only use the points displayed as input to the calculation.

#### Exercise 12 – Calculate with definition query

- Make sure only the western part of the Area of Interest has been selected

- Right click North Atlantis Baseline and select Open Attribute Table

- Notice that there is a column called Region. We will use this when set up the definition query

- Right click North Atlantis Baseline and select Properties

- Click the Definition Query tab

- Click Query Builder …

- In the list at the top, double click Region

- Click  =

- Click Get Unique Values, and a list of possible values will pop up

- Double click ‘B’

- Click OK

- Click OK again

- Notice that only baseline segments with Region name B is shown in the map.

- Double click the Calculate Limit Line tool

- From the dropdown menu select North Atlantis Baseline

- From the Baseline Type Column dropdown menu select Type

- From the Straight Baseline Attribute dropdown menu select Straight

- Under Distance type in 12

- Under Calculation Area polygon select Area of Interest

- Under Output name type in TS

- Leave all other options as default and click OK

- Once the tool finishes, notice that the resulting line is only calculated from the points specified in the definition query

Make sure you remove the definition query again so that the whole baseline is used the next time we use it as input.

## Multiple input calculation

Many organizations will have multiple basepoint datasets, split by region or basepoint type (normal, straight, bay-closing, islands etc.). In these cases, we can use the Calculate Limit Line Multiple Input tool to use multiple baseline datasets as input to the calculation. The tool itself is the same as Calculate Limit Line, except for the dataset input part.

#### Exercise 13 – Calculate from multiple input datasets

- Make sure the southern and eastern Area of Interest polygons have been selected

- Double click the Calculate Limit Line Multiple Input tool

- From the Normal Baselines dropdown menu select South Atlantis Baseline

- From the Straight Baselines dropdown menu select East Atlantis Baseline

- Under Distance type in 12

- Under Calculation Area polygon select Area of Interest

- Under Output name type in TS

- Leave all other options as default and click OK

- Once the tool finishes notice that all datasets have been used as input to the calculation

- Inspect the resulting data and observe the differences

# Equidistance lines

The Calculate Equidistant Line tool calculates an equidistant line between a coastal state and its neighboring countries. The distance can optionally be weighed according to a given attribute field in the baseline feature class. The tools use geodesic operations and the resulting limit line will have a high accuracy.

## Input

#### Exercise 1 – Investigate the input parameters of the Calculate Equidistant Line tool

- Double click the Calculate Equidistant Line tool

- Click Show Help >>

- Click on each input element and read the help text

- Click Cancel

## Output

The Calculate Equidistant Line tool offer four different output datasets. It is up to the user to decide which output dataset to generate. The name of the resulting dataset will use the user defined output name as the prefix and the i.e _Midline

Read more about the different output datasets below:

Output Midline
The midline consists of a single line feature made up by the midpoints.

Output Midpoints
The Midpoints feature class will have one feature for each point on the mid-line. The Attribute table of a midpoints feature class will have columns for the latitude and longitude of the point. The point type, distance to the base points, and information of weights has been applied to any of the base point distances.

Output Tripoints
Tripoints are the "turning points" of the equidistant line. These are the points on the equidistant line which have the same distance to three base points. The geometry if the tri points are three line pieces from the tri point to each of the three base points. The attribute table of the tripoints will have columns for latitude and longitude of the tripoint, and latitude and longitude of all three base point. It will also have columns for distances to all base points.

Output Connection Polygon
The connection polygon is used to visualize the association between the midline and the critical baseline points. In the connection polygon feature class, each feature is a combination of the a midline piece, and the associated base line point. The geometry of the features will look like triangles where one corner of the triangle is the base line, and the opposite edge will be the midline where this base point is the nearest base points to the equidistant line.

## Straight baselines

When working with straight baselines during distance calculation the densification of straight baseline was done automatically and behind the scenes. When generating equidistance lines, this must be done as a pre-step by the user, otherwise only the vertices at each end of the line will be taken into account.

#### Exercise 2 – Geodetic densify East Atlantis

- Open the tool Geodetic Densify which you will find under Data Management Tools and Features

- Under Input Features select East Atlantis Baseline

- Under Output Feature Class browse for the Atlantis_MLB database and double click it

- Type in the name East_Atlantis_Densify

- Click Save

- Under Geodetic Type make sure GEODESIC has been selected

- Under Distance set 0,5 and set det unit dropdown to Nautical Miles

- Click OK

Once the tool finishes the densified line will be added to the map. Although you won`t see a big difference compared to the original, it does consist of a lot more points.

Since the North Atlantis Baseline consist of both normal and straight segments we want to make sure we only densify the straight segments. This can be done by selecting them before the densification process.

#### Exercise 3 – Geodetic densify North Atlantis

- Right click North Atlantis Baseline and select Open Attribute Table

- Select all straight segments by holding down Ctrl on your keyboard and clicking.

- Open the tool Geodetic Densify which you will find under Data Management Tools and Features

- Under Input Features select North Atlantis Baseline

- Under Output Feature Class browse for the Atlantis_MLB database and double click it

- Type in the name North_Atlantis_Densify

- Click Save

- Under Geodetic Type make sure GEODESIC has been selected

- Under Distance set 0,5 and set det unit dropdown to Nautical Miles

- Click OK

- The resulting dataset will consist of only the straight segments, which we now have to merge together with the original baseline.

- Make sure your clear the selection of the straight baseline segments, so nothing is selected

- Open the tool Merge which you will find under Data Management Tools and General

- Under Input Features select North_Atlantis_Densify

- Then under Input Features again, select North Atlantis Baseline

- Under Output Dataset browse for the Atlantis_MLB database and double click it

- Type in the name North_Atlantis_Merge

- Click OK

- Once the tool finishes, right click North_Atlantis_Merge and select Open Attribute Table

- The dataset will consist of both the original straight segments and the densified straight segments (at the top of the table). We need to delete the original segments before we continue.

- Right click North_Atlantis_Merge, go to Edit Features and click Start Editing

- Click Continue

- Select the 4 original straight segments (not the first 4 in the table)

- Click Delete Selected  on the Attribute Table toolbar

- On the Editor toolbar select Save Edits

- On the Editor toolbar select Stop Editing

## We are now ready to start calculating equidistance lines

## Basic calculation

#### Exercise 4 – Simple midline generation

- Select the to western most polygons of the Area of Interest

- Double click Calculate Equidistant Line

- From the Main Country Baseline Feature Class dropdown select North_Atlantis_Merge

- From the Neighbouring Country Baseline Feature Classes dropdown select South Atlantis  Baseline

- Under Calculation Area polygon select Area of Interest (make sure nothing has been selected)

- Under Output Name type in North_midline

- Leave all other options as default and click OK

#### Exercise 5 – Inspect the results

- Turn of the Area of Interest

- Turn off the results called North_midline_Connection_Polygons and North_midline_Tri_Points

- Notice that you have one dataset for the equidistance line and one for the actual turningpoints/midpoints

- Right click North_midline_Mid_Points and click Open Attribute Table

- Notice that Latitude and Longitude is provided for each turning point, as well as the distance to each contributing basepoint.

- Turn on North_midline_Tri_Points and notice that you can now see which basepoints contributes to the generation of each turning point

- Right click North_midline_Tri_Points and click Open Attribute Table

- Notice that in this table you get the longitude and latitude for each contributing basepoint

- Select one of the rows in the table in order to clearly see which points contribute to a certain turningpoint

## Multiple states calculations

It is possible to use multiple States as input to an equidistance calculation. In the exercise we are going to calculate the midline between East Atlantis and North and South Atalantis

#### Exercise 6 – Calculate between multiple states

- Double click Calculate Equidistant Line

- From the Main Country Baseline Feature Class dropdown select East_Atlantis_Densify

- From the Neighbouring Country Baseline Feature Classes dropdown select North_Atlantis_Merge and South Atlantis Baseline

- Under Calculation Area polygon select Area of Interest (make sure an area has not been selected)

- Under Output Name type in East_Midline

- Leave all other options as default and click OK

- Notice the difference at the beginning of the line and how the line is constructed based on basepoint from both neighboring states

- Try finding the point which is an equal distance between all three states

## Weighted calculations

In some cases, small features likes islands etc. can give a disproportionate effect to the final midline. In these cases it is possible to apply less weight to these features. In the next exercise we will give the island on North Atlantis only 50% weight.

#### Exercise 7 – Add weights to basepoints

- Right click North_Atlantis_Merge and click Open Attribute Table

- Click on Table Options icon  and click Add Field...

- In Name type in Weight

- In Type select Double

- Click OK

- Right click then new Weight column and select Field Calculator

- Click Yes

- Type in 1 in the text box (this gives all basepoints full (1) weight)

- Click OK

- Notice that all points now have the weight 1

- Make North_Atlantis_Merge the only selectable layer

- Zoom into the north eastern island of North_Atlantis_Merge and select the portion on the baseline which resides on the island

- Right click the Weight column and select Field Calculator

- Click Yes

- Type in 0.5 in the text box

- Click OK

The selected points now have the weight 0.5, while the remaining baseline points have full weight.

#### Exercise 8 – Calculate with weights

- On the main Toolbar click the Clear selection button

- Double click Calculate Equidistant Line

- From the Main Country Baseline Feature Class dropdown select East_Atlantis_Densify

- From the Neighbouring Country Baseline Feature Classes dropdown select of North_Atlantis_Merge and South Atlantis Baseline

- From the Weights dropdown select the field Weight

- Under Calculation Area polygon select Area of Interest

- Under Output Name type in East_Weighted

- Leave all other options as default and click OK

Once the tool finishes compare the two equidistant lines East_Weighted and East_Midline you have computed so far is displayed in the map, in addition to the input data.

## Area calculation

Should you want to calculate an area of a polygon in ArcGIS it is important to know that the Calculate Geometry tool calculates area in the coordinate system of the data source or data frame.

To calculate a geodesic area, you should use the ArcPy geometry objects. This can be done through the Python parser in the Field Calculator. Use the following expression:

!Shape!.getArea("GEODESIC","SQUAREKILOMETERS")